# NVMe Namespace Consolidation

nvme-cli

# The Problem:

- When NVMe (usually datacenter “DC”) devices are enumerated as multiple drives of varying sizes, they have been divided by namespace into separate logical drives.

- NVMe drives can also fail to enumerate to the operating system if there are no namespaces (disks) created on the device, or if they have not been attached to the controller.

# The Solution:

- Use nvme-cli in Ubuntu in order to delete the current namespaces and then recreate a new name space using the entire drive capacity.

- If devices show in (lspci | grep -i nvme) or (ls /dev/nvme*) the device is working and probably requires namespaces to be created and attached to the controller in order to be seen as a storage disk.

# The Process:

- Boot up linux and make sure nvme-cli is installed.

  - sudo apt install nvme-cli

- List NVMe Devices (FW, Serial, Model, Controller, Namespace, Capacity, Sector)

  - sudo nvme list

- Identify accepted LBA format of namespace, (default usually format “0” AKA 512)

  - sudo nvme id-ns /dev/nvme0n1 -H | grep -i "LBA Format"

- Identify the sizes of the first 3 namespaces total !!BEFORE DELETION!!

Note: Scripts are included later in the document for this purpose.

  - namespace1=$(sudo nvme id-ns /dev/nvme0 -n 1 | awk '/nsze/ {print $3}')

namespace2=$(sudo nvme id-ns /dev/nvme0 -n 2 | awk '/nsze/ {print $3}')

namespace3=$(sudo nvme id-ns /dev/nvme0 -n 3 | awk '/nsze/ {print $3}')

echo "namespace1=$namespace1  namespace2=$namespace2 namespace3=$namespace3"

total=$(( namespace1 + namespace2 + namespace3 ))

echo "total nsze = $total"

- Delete the existing namespaces

  - sudo nvme delete-ns /dev/nvme0 -n 1

  - sudo nvme delete-ns /dev/nvme0 -n 2

  - sudo nvme delete-ns /dev/nvme0 -n 3

- Create the new total namespace using total LBA size (6.4TB = 12501123072 LBA)

  - sudo nvme create-ns /dev/nvme0 -s 12501123072 -c 12501123072 -f 0

- List controllers on drive (default controller = 0)

  - sudo nvme list-ctrl /dev/nvme0

- Attach the new namespace to the controller (default = 0)

  - sudo nvme attach-ns /dev/nvme0 -n 1 -c 0

- Refresh the connection to the drive

  - sudo nvme reset /dev/nvme0

- Verify drive capacity

  - sudo nvme list or lsblk

- Drives should now enumerate as a single device with its total capacity.

# NVMe-CLI Commands:

- sudo nvme list							# lists all nvme drives

- sudo nvme list-ns <device>					# list name spaces

- sudo nvme id-ns <device> <-n #> -H				# info on namespace

- sudo nvme list-ctrl <device>					# list controller id’s

- sudo nvme id-ctrl <device> -H					# info on controller

- sudo nvme delete-ns <device> <-n #>				# delete namespace

- sudo nvme create-ns <device> <nsze> <cap> <format>	# create namespace

  - -s <LBA total>					## namespace size in LBA

  - -c <LBA total>					## namespace capacity in LBA

  - -f 0						## format sector size - 512

  - -f 1						## format sector size - 4096

- sudo nvme id-ns /dev/nvme0n1 -H | grep -i "LBA Format"   	# get LBA format type

- sudo nvme attach-ns <device> <namespace> <controller> 	# attach to controller

  - -n 1						## namespace 1

  - -c 0						## controller 0

- sudo nvme reset /dev/nvme0					# reset for enumeration

***Don’t forget to chmod +x <scriptname> in order to add execution privileges***

## Script - Check for Controllers

./check_controllers.sh or ls /dev/nvme*

For when drives don’t show due to having no namespaces, check for the controller.

#!/usr/bin/env bash

for ctrl in /dev/nvme[0-9]*; do

echo "=== $ctrl ==="

ns_out=$(sudo nvme list-ns "$ctrl" -a 2>/dev/null)

if [[ -z "$ns_out" ]]; then

echo "  No namespaces found (nvme list-ns -a returned nothing)."

else

echo "$ns_out" | sed 's/^/  /'

fi

echo

done

## Script - Add Namespace LBA’s (Primitive)

./add_namespace_LBA.sh

Get first 3 namespace LBA sizes & add them together for total namespace creation.

#!/usr/bin/env bash

namespace1=$(sudo nvme id-ns /dev/nvme0 -n 1 | awk '/nsze/ {print $3}')

namespace2=$(sudo nvme id-ns /dev/nvme0 -n 2 | awk '/nsze/ {print $3}')

namespace3=$(sudo nvme id-ns /dev/nvme0 -n 3 | awk '/nsze/ {print $3}')

echo "namespace1=$namespace1  namespace2=$namespace2 namespace3=$namespace3"

total=$(( namespace1 + namespace2 + namespace3 ))

echo "total nsze = $total"

## Script - Calculating Total Namespaces LBA

sudo ./sum_nsze.sh /dev/nvme0

Get size of all name spaces, combine to total

#!/usr/bin/env bash

set -euo pipefail

# Usage: ./sum_nsze.sh /dev/nvme0

# Default controller: /dev/nvme0 if none passed

ctrl=${1:-/dev/nvme0}

echo "Using controller: $ctrl"

echo "Collecting namespaces from nvme list-ns ..."

# Get raw output once

raw_output=$(sudo nvme list-ns "$ctrl" -a 2>/dev/null || true)

printf '%s\n' "$raw_output"

# Parse nsids from possible formats:

#   1

#   nsid: 1

#   [   0]:0x1  (your case)

mapfile -t nsids < <(

printf '%s\n' "$raw_output" | awk '

# case 1: plain decimal line like "1"

/^[0-9]+$/ { print $1; next }

# case 2: verbose "nsid: 1"

$1 == "nsid:" && $2 ~ /^[0-9]+$/ { print $2; next }

# case 3: "[   0]:0x1" style -> extract the 0x... part

/^\[/ {

if (match($0, /0x[0-9A-Fa-f]+/)) {

print substr($0, RSTART, RLENGTH)

}

next

}

'

)

if [[ ${#nsids[@]} -eq 0 ]]; then

echo

echo "!!! No namespaces parsed from nvme list-ns output."

echo "Check the above raw output; parser may need tweaking for this system."

exit 1

fi

echo

echo "Parsed namespace IDs (raw): ${nsids[*]}"

total=0

for raw_nsid in "${nsids[@]}"; do

# Convert nsid to decimal (handles 0x1 style)

if [[ $raw_nsid == 0x* || $raw_nsid == 0X* ]]; then

nsid=$((16#${raw_nsid#0x}))

else

nsid=$raw_nsid

fi

echo

echo "Processing namespace: raw_nsid=$raw_nsid  ->  nsid=$nsid"

# Get nsze (may be decimal or 0xHEX)

raw_nsze=$(sudo nvme id-ns "$ctrl" -n "$nsid" | awk '/nsze/ {print $3}')

if [[ $raw_nsze == 0x* || $raw_nsze == 0X* ]]; then

nsze=$((16#${raw_nsze#0x}))

else

nsze=$raw_nsze

fi

echo "  nsze=$nsze"

total=$(( total + nsze ))

done

echo

echo "Total nsze across all namespaces on $ctrl = $total (LBAs)"

echo

echo "Example create-ns command:"

echo "  sudo nvme create-ns $ctrl -s $total -c $total -f 0"

## Script - Recombine All NVMe Namespaces

sudo ./recombine_nvme_namespaces.sh -d /dev/nvme0

Get LBA’s, Detach all ns, Delete all ns, Create total ns, Attach ns to controller 0, refresh, list

#!/usr/bin/env bash

set -euo pipefail

usage() {

cat <<EOF

Usage: sudo $0 [options]

Options:

-d, --device <ctrl>   NVMe controller device (default: /dev/nvme0)

-F, --format <idx>    LBA format index to use for new namespace (default: 0)

-y, --force           Don't prompt for confirmation

-h, --help            Show this help

Example:

sudo $0 -d /dev/nvme0 -F 0

EOF

}

ctrl="/dev/nvme0"

format=0

force=0

# --- Parse arguments ---

while [[ $# -gt 0 ]]; do

case "$1" in

-d|--device)

ctrl="$2"

shift 2

;;

-F|--format)

format="$2"

shift 2

;;

-y|--force)

force=1

shift

;;

-h|--help)

usage

exit 0

;;

*)

echo "Unknown option: $1"

usage

exit 1

;;

esac

done

if [[ ! -e "$ctrl" ]]; then

echo "Controller $ctrl does not exist."

exit 1

fi

echo "Using controller: $ctrl"

echo

# --- Step 1: List namespaces (raw) ---

echo "Collecting namespaces from: nvme list-ns $ctrl -a"

raw_output=$(sudo nvme list-ns "$ctrl" -a 2>/dev/null || true)

printf '%s\n' "$raw_output"

echo

# Parse nsids from possible formats:

#   1

#   nsid: 1

#   [   0]:0x1   (your case)

mapfile -t nsids < <(

printf '%s\n' "$raw_output" | awk '

# case 1: plain decimal line like "1"

/^[0-9]+$/ { print $1; next }

# case 2: verbose "nsid: 1"

$1 == "nsid:" && $2 ~ /^[0-9]+$/ { print $2; next }

# case 3: "[   0]:0x1" style -> extract the 0x... part

/^\[/ {

if (match($0, /0x[0-9A-Fa-f]+/)) {

print substr($0, RSTART, RLENGTH)

}

next

}

'

)

if [[ ${#nsids[@]} -eq 0 ]]; then

echo "No namespaces parsed from nvme list-ns output."

echo "Nothing to do."

exit 1

fi

echo "Parsed namespace IDs (raw): ${nsids[*]}"

echo

# --- Step 2: Sum nsze for all namespaces ---

total=0

first_nsid_dec=""

for raw_nsid in "${nsids[@]}"; do

# Convert nsid to decimal (handles 0x1 style)

if [[ $raw_nsid == 0x* || $raw_nsid == 0X* ]]; then

nsid=$((16#${raw_nsid#0x}))

else

nsid=$raw_nsid

fi

[[ -z "$first_nsid_dec" ]] && first_nsid_dec="$nsid"

echo "Processing namespace: raw_nsid=$raw_nsid -> nsid=$nsid"

raw_nsze=$(sudo nvme id-ns "$ctrl" -n "$nsid" | awk '/nsze/ {print $3}')

if [[ $raw_nsze == 0x* || $raw_nsze == 0X* ]]; then

nsze=$((16#${raw_nsze#0x}))

else

nsze=$raw_nsze

fi

echo "  nsze=$nsze"

total=$(( total + nsze ))

done

echo

echo "Total nsze across all namespaces on $ctrl = $total (LBAs)"

# --- Optional: show approximate size in bytes/GiB for sanity ---

# Try to derive LBA size from first namespace

lba_bytes=""

if [[ -n "$first_nsid_dec" ]]; then

lbads_line=$(sudo nvme id-ns "$ctrl" -n "$first_nsid_dec" | awk '/lbads/ {print $0}' || true)

lbads=$(printf '%s\n' "$lbads_line" | awk '{print $NF}' || true)

if [[ "$lbads" =~ ^[0-9]+$ ]]; then

lba_bytes=$(( 1 << lbads ))

bytes=$(( total * lba_bytes ))

gib=$(( bytes / 1024 / 1024 / 1024 ))

echo "Approximate combined size: ${bytes} bytes (~${gib} GiB) using lbads=${lbads}"

fi

fi

echo

echo "!! WARNING: This will DELETE ALL EXISTING NAMESPACES on $ctrl"

echo "and create ONE new namespace of size $total LBAs using format index $format."

echo

if [[ $force -eq 0 ]]; then

read -rp "Type EXACTLY 'YES' to proceed: " ans

if [[ "$ans" != "YES" ]]; then

echo "Aborted by user."

exit 1

fi

else

echo "--force specified, skipping confirmation."

fi

echo

echo "--- Step 3: Detaching namespaces (best effort) ---"

# Get controller IDs (cntlid) for detach/attach

mapfile -t cntlids < <(sudo nvme list-ctrl "$ctrl" 2>/dev/null | awk '

/cntlid/ {

for (i = 1; i <= NF; i++) {

if ($i ~ /^[0-9]+$/) { print $i; break }

}

}

')

if [[ ${#cntlids[@]} -eq 0 ]]; then

echo "No controller IDs parsed from nvme list-ctrl. Assuming cntlid 0."

cntlids=(0)

else

echo "Controller IDs: ${cntlids[*]}"

fi

# Detach each nsid from each controller ID (ignore errors)

set +e

for raw_nsid in "${nsids[@]}"; do

if [[ $raw_nsid == 0x* || $raw_nsid == 0X* ]]; then

nsid=$((16#${raw_nsid#0x}))

else

nsid=$raw_nsid

fi

for cid in "${cntlids[@]}"; do

echo "Detaching nsid=$nsid from cntlid=$cid (ignore errors)..."

sudo nvme detach-ns "$ctrl" -n "$nsid" -c "$cid" >/dev/null 2>&1 || true

done

done

set -e

echo

echo "--- Step 4: Deleting namespaces ---"

for raw_nsid in "${nsids[@]}"; do

if [[ $raw_nsid == 0x* || $raw_nsid == 0X* ]]; then

nsid=$((16#${raw_nsid#0x}))

else

nsid=$raw_nsid

fi

echo "Deleting namespace nsid=$nsid ..."

sudo nvme delete-ns "$ctrl" -n "$nsid"

done

echo

echo "Verifying no namespaces remain..."

post_del_output=$(sudo nvme list-ns "$ctrl" -a 2>/dev/null || true)

printf '%s\n' "$post_del_output"

if [[ -n "$post_del_output" ]]; then

echo "WARNING: nvme list-ns still shows data; controller may behave oddly."

echo "Continuing anyway with create-ns..."

fi

echo

echo "--- Step 5: Creating a single large namespace ---"

create_out=$(sudo nvme create-ns "$ctrl" -s "$total" -c "$total" -f "$format")

echo "$create_out"

# Parse new nsid from create-ns output, e.g. "create-ns: Success, created nsid:1"

new_raw_nsid=$(printf '%s\n' "$create_out" | awk '

{

# Look for "nsid:<number>" in the line

if ($0 ~ /nsid:[0-9]+/) {

line = $0

# Strip everything up to and including "nsid:"

sub(/.*nsid:/, "", line)

# Strip anything after the digits (just in case)

sub(/[^0-9].*$/, "", line)

print line

exit

}

}

')

if [[ -z "$new_raw_nsid" ]]; then

echo "ERROR: Could not parse new namespace ID from create-ns output."

exit 1

fi

if [[ $new_raw_nsid == 0x* || $new_raw_nsid == 0X* ]]; then

new_nsid=$((16#${new_raw_nsid#0x}))

else

new_nsid=$new_raw_nsid

fi

echo "New namespace ID: raw=$new_raw_nsid -> nsid=$new_nsid"

echo

echo "--- Step 6: Attaching new namespace to controller ---"

attach_cid="${cntlids[0]}"

echo "Attaching nsid=$new_nsid to cntlid=$attach_cid ..."

sudo nvme attach-ns "$ctrl" -n "$new_nsid" -c "$attach_cid"

echo

echo "--- Step 7: Resetting controller ---"

sudo nvme reset "$ctrl"

echo

echo "--- Done. Current NVMe devices: ---"

sudo nvme list

echo

echo "You should now see a single large namespace for $ctrl (e.g., ${ctrl}n1)."