Thermal Management on Dell R730xd

w/ 3rd Party PCIE Cards

# Why:

When testing 3rd party PCIE cards that are not official Dell cards, they will spin fans at 100% speed as a safety precaution. If the card does not present an overheating risk or cooling issue, you can download OpenManage Server Administrator (OMSA) and modify the fan settings. Simply changing the power profile in BIOS will only work pre-OS. Once the OS loads it will spin fans at maximum speed.

# Requirements:

Dell EMC OpenManage Server Administrator Managed Node for Windows v10.3.0.0

# Hardware:

Dell PowerEdge R730xd

Any 3rd Party PCIE cards

# How To:

- Go to Dell Support -> Drivers & Downloads -> PowerEdge R730xd -> System Management

  - https://www.dell.com/support/home/en-us

- Find “OpenManage Server Administrator Node for Windows” and Download, it is required to use racadm commands via command line.

- Extract the ZIP file (C:\OpenManage\ by default)

- Go to the windows folder and run the setup.exe

- Proceed with a Full Installation (includes Storage & System components)

- After the installation is complete, open up windows Services and ensure these are running:

  - DSM SA Data Manager

  - DSM SA Connection Service

- On the server itself go to your browser and open https://localhost:1311

  - Accept any SSL certificate warnings

  - Log in using the server's Administrator credentials.

- Firewall check (should add rule automatically)

  - Open PowerShell and run:

    - New-NetFirewallRule -DisplayName “OMSA” -Direction Inbound -Protocol TCP -LocalPort 1311 -Action Allow

- Modify thermal settings:

  - Open Command Prompt (Administrator) and run:

    - racadm set system.thermalsettings.ThirdPartyPCIFanResponse 0

# Remote Management (Optional)

- On the Dell Server, spin up a webserver in PowerShell by running:

  - omconfig preferences webserver attribute=ipaddress setting=ALL

- Restart these services in Command Prompt (Administrator)

  - net stop "DSM SA Connection Service"

  - net stop "DSM SA Data Manager"

  - net start "DSM SA Data Manager"

  - net start "DSM SA Connection Service"

- From the other machine:

  - Go to https://<server.ip.address>:1311 (not the iDRAC ip)

- Log-in using administrator credentials

- Manage the server using the GUI